Heads up: the netlist you gave is not an AES core—it’s a small “done fan-in” block with four flip-flops (dcnt[3:0]) and a done output. Below is a clean behavioral RTL that is functionally equivalent to your netlist, followed by a compact signal-correlation map.

Behavioral RTL (equivalent)

`timescale 1ns/1ps

module done_fanin (

input  wire rst,   // synchronous enable/gate in this design

input  wire ld,    // load/control bit

input  wire clk,

output reg  done

);

reg dcnt0, dcnt1, dcnt2, dcnt3;

// convenience

wire any_one = dcnt0 | dcnt1 | dcnt2 | dcnt3;

// Synchronous next-state logic (simplified from the gate cones)

always @(posedge clk) begin

// When rst=0, all flops synchronously drive to 0 (since every D has a factor of rst)

dcnt0 <= rst & (  ld | ( ~ld & any_one & ~dcnt0 ) );

dcnt1 <= rst & (  ld | ( ~ld & any_one & ~(dcnt0 ^ dcnt1) ) );

dcnt2 <= rst & ( ~ld & dcnt2 & (dcnt1 | dcnt0) );

dcnt3 <= rst & (  ld | ( ~ld & dcnt3 & (dcnt2 | dcnt1 | dcnt0) ) );

// done is high exactly when ld=0 and dcnt == 4'b0001

done  <= (~dcnt3) & (~dcnt2) & (~dcnt1) & dcnt0 & (~ld);

end

endmodule

What it does (in plain terms)

- rst acts like a synchronous gate/enable: when rst=0, all four registers are driven to 0 on the next clock; when rst=1, the logic below takes effect.

- With rst=1 and ld=1, the next state is forced to a fixed pattern (due to the | ld terms on three bits); with ld=0, the four bits evolve according to simple relations involving the current dcnt value.

- done asserts iff ld=0 and the register pattern is exactly dcnt = 4'b0001.

Correlation map (netlist ↔ behavioral)

Flip-flops / state:

- dcnt_reg_0_ → dcnt0 (QN n9179 = ~dcnt0)

- dcnt_reg_1_ → dcnt1 (Q n4590 = dcnt1, QN n9178 = ~dcnt1)

- dcnt_reg_2_ → dcnt2 (QN n9042 = ~dcnt2)

- dcnt_reg_3_ → dcnt3 (Q n1774 = dcnt3, QN n9041 = ~dcnt3)

- done_reg → done (D = N23)

Key simplified internal relations (from your gates):

- n1771 = ~(~dcnt1 & ~dcnt0) = (dcnt1 | dcnt0)

- n1767 = ~n1771 = (~dcnt1 & ~dcnt0)

- n1764 = ~(~dcnt2 & n1767) = (dcnt2 | dcnt1 | dcnt0)

- n1775 = (dcnt3 | n1764) = (dcnt3 | dcnt2 | dcnt1 | dcnt0) = any_one

- n1768 = rst & ~ld & any_one

- n1770 = rst & ld

- n1769 = ~((~dcnt0 | ~dcnt1) & (dcnt1 | dcnt0)) = ~(dcnt0 ^ dcnt1) (XNOR)

Next-state pins (exactly matching D inputs, but simplified):

- D0 (n9184) = rst & ( ld | (~ld & any_one & ~dcnt0) )

- D1 (n9182) = rst & ( ld | (~ld & any_one & ~(dcnt0 ^ dcnt1)) )

- D2 (n9181) = rst & (~ld & dcnt2 & (dcnt1 | dcnt0))

- D3 (n9183) = rst & ( ld | (~ld & dcnt3 & (dcnt2 | dcnt1 | dcnt0)) )

done path:

- n4589 = ~(~dcnt3 & ~dcnt2) = (dcnt3 | dcnt2)

- N23 = ~( n4589 | dcnt1 | ~dcnt0 | ld ) → done = (~dcnt3)&(~dcnt2)&(~dcnt1)&dcnt0&(~ld)

Inverter chains on ld in the netlist (n9474, n9473, n9472, n9446, n9437, n9471, n9448, n9468, n9467) all collapse to either ld or ~ld in the simplified form and are reflected above as such.